# NOTIFICATION

The following notification is being circulated in accordance with Article 10.6

| 1. | Notifying Member: UGANDA If applicable, name of local government involved (Article 3.2 and 7.2): |
| --- | --- |
| 2. | Agency responsible: Uganda National Bureau of Standards Plot 2-12 ByPass Link, Bweyogerere Industrial and Business Park P.O. Box 6329 Kampala, Uganda Tel: +(256) 4 1733 3250/1/2 Fax: +(256) 4 1428 6123 E-mail: info@unbs.go.ug Website: https://www.unbs.go.ug Name and address (including telephone and fax numbers, email and website addresses, if available) of agency or authority designated to handle comments regarding the notification shall be indicated if different from above: |
| 3. | Notified under Article 2.9.2 [ ], 2.10.1 [ ], 5.6.2 [X], 5.7.1 [ ], 3.2 [ ], 7.2 [ ], other: |
| 4. | Products covered (HS or CCCN where applicable, otherwise national tariff heading. ICS numbers may be provided in addition, where applicable): Petroleum gases and other gaseous hydrocarbons. (HS code(s): 2711); Fuels (ICS code(s): 75.160), Gaseous Hydrocarbons, Liquefied Petroleum Gases, Natural Gas |
| 5. | Title, number of pages and language(s) of the notified document: DUS 2527:2022, Standard Test Method for Determination of Total Volatile Sulfur in Gaseous Hydrocarbons and Liquefied Petroleum Gases and Natural Gas by Ultraviolet Fluorescence, First Edition; (17 page(s), in English) |
| 6. | Description of content: This Draft Uganda Standard covers the determination of total volatile sulfur in gaseous hydrocarbons, Liquefied Petroleum Gases (LPG) and Liquefied Natural Gas (LNG). It is applicable to analysis of natural gaseous fuels, process intermediates, final product hydrocarbons and generic gaseous fuels containing sulfur in the range of 1 to 200 mg/kg. Samples can also be tested at other total sulfur levels using either pre-concentration methods or sample dilution using a diluent gas. The methodology for preconcentration and dilution techniques is not covered in this test method. The precision statement does not apply if these techniques are used in conjunction with this test method. The diluent gas, such as UHP nitrogen, zero nitrogen or zero air, shall not have a significant total sulfur concentration. This test method may not detect sulfur compounds that do not volatilize under the conditions of the test. This test method covers the laboratory determination and the at-line/on-line determination of total volatile sulfur in gaseous fuels, LPG, and LNG. This test method is applicable for total volatile sulfur determination in gaseous hydrocarbons, LPG, and LNG containing less than 0.35 mole % halogen(s). |
| 7. | Objective and rationale, including the nature of urgent problems where applicable: Prevention of deceptive practices and consumer protection; Quality requirements |
| 8. | Relevant documents: - ASTM D1070 Test Methods for Relative Density of Gaseous Fuels - ASTM D1072 Test Method for Total Sulfur in Fuel Gases by Combustion and Barium Chloride Titration - ASTM D1265 Practice for Sampling Liquefied Petroleum (LP) Gases, Manual Method - ASTM D3588 Practice for Calculating Heat Value, Compressibility Factor, and Relative Density of Gaseous Fuels - ASTM D3609 Practice for Calibration Techniques Using Permeation Tubes - ASTM D4150 Terminology Relating to Gaseous Fuels - ASTM D4177 Practice for Automatic Sampling of Petroleum and Petroleum Products - ASTM D4784 Specification for LNG Density Calculation Models - ASTM D5287 Practice for Automatic Sampling of Gaseous Fuels - ASTM D5503 Practice for Natural Gas Sample-Handling and Conditioning Systems for Pipeline Instrumentation - ASTM D5504 Test Method for Determination of Sulfur Compounds in Natural Gas and Gaseous Fuels by Gas Chromatography and Chemiluminescence - ASTM D6228 Test Method for Determination of Sulfur Compounds in Natural Gas and Gaseous Fuels by Gas Chromatography and Flame Photometric Detection - ASTM D6299 Practice for Applying Statistical Quality Assurance and Control Charting Techniques to Evaluate Analytical Measurement System Performance - ASTM D7166 Practice for Total Sulfur Analyzer Based On-line/At-line for Sulfur Content of Gaseous Fuels - ASTM E617 Specification for Laboratory Weights and Precision Mass Standards - ASTM E691 Practice for Conducting an Interlaboratory Study to Determine the Precision of a Test Method - ASTM F307 Practice for Sampling Pressurized Gas for Gas Analysis - ASTM MNL 7 - GPA 2166 Obtaining Natural Gas Samples for Analysis by Gas Chromatography - GPA 2174 Obtaining Liquid Hydrocarbon Samples for Analysis by Gas Chromatography |
| 9. | Proposed date of adoption: To be determined Proposed date of entry into force: Not applicable. |
| 10. | Final date for comments: 60 days from notification |
| 11. | Texts available from: National enquiry point [X] or address, telephone and fax numbers and email and website addresses, if available, of other body: Uganda National Bureau of Standards Plot 2-12 ByPass Link, Bweyogerere Industrial and Business Park P.O. Box 6329 Kampala, Uganda Tel: +(256) 4 1733 3250/1/2 Fax: +(256) 4 1428 6123 E-mail: info@unbs.go.ug Website: https://www.unbs.go.ug |