# NOTIFICATION

The following notification is being circulated in accordance with Article 10.6.

| 1. | Notifying Member: CABO VERDE If applicable, name of local government involved (Articles 3.2 and 7.2): |
| --- | --- |
| 2. | Agency responsible: Ministère de l'Industrie, Commerce et Energie (Ministry of Industry, Trade and Energy) Name and address (including telephone and fax numbers, email and website addresses, if available) ofagency or authority designated to handle comments regarding the notification shall be indicated if different from above: Direção Nacional da Indústria, Comércio e Energia (National Directorate for Industry, Trade and Energy) Point d'information sur les OTC (TBT Enquiry Point) Telephone: (+238) 2604800 Email: benvindo.reis@mice.gov.cv or jonica.tavares@mice.gov.cv |
| 3. | Notified under Article 2.9.2 [X], 2.10.1 [ ], 5.6.2 [ ], 5.7.1 [ ], 3.2 [ ], 7.2 [ ], other: |
| 4. | Products covered (HS or CCCN where applicable, otherwise national tariff heading. ICS numbers may be provided in addition, where applicable): Butane for the domestic market (HS code(s): 2711.13) |
| 5. | Title, number of pages and language(s) of the notified document: Ordonnance conjointe nº 73/2005 du 26 décembre 2005, établissant les spécifications techniques du butane (Joint Order No. 73/2005 of 26 December 2005 establishing technical specifications for butane) (5 pages, in Portuguese) |
| 6. | Description of content: The notified Order establishes the technical requirements for butane, including: Physico-chemical properties: - Density at 15°C (min. 0.560 kg/m³, ISO standard 3993) - Composition (C4 ≥ 85% molar mass, EN 27941) - Maximum total sulphur content (50 mg/kg, EN 24260) Safety and quality: - Vapour pressure at 40°C (max. 520 kPa, EN ISO 24256) - Copper strip corrosion (max. 1, ASTM D 1838). - Absence of water, separated or in suspension (visual inspection) Energy performance: Net calorific value (min. 10,000 kcal/kg, ASTM D 3588) Test methods: ISO, EN, ASTM and NP standards specified for each parameter |
| 7. | Objective and rationale, including the nature of urgent problems where applicable: - Ensure the quality and safety of marketed butane - Harmonize specifications with international standards (ISO, EN) - Protect consumers and the environment (limit on pollutants) Protection of the environment; Harmonization |
| 8. | Relevant documents: https://kb-wordpress.gov.cv/kb/portaria-no-73-2005-especificacoes-do-butano/ |
| 9. | Proposed date of adoption: 26 December 2005 Proposed date of entry into force: 26 December 2005 |
| 10. | Final date for comments: Not applicable; Regulation already in force, which had not been notified in time. |
| 11. | Texts available from: National enquiry point [ ] or address, telephone and fax numbers and email and website addresses, if available, of other body: Direção Nacional da Indústria, Comércio e Energia (National Directorate for Industry, Trade and Energy) Direction du Service de l'Energie (Energy Service Directorate) or Point d'information sur les OTC (TBT Enquiry Point) Telephone: (+238) 2604800 Email: benvindo.reis@mice.gov.cv or jonica.tavares@mice.gov.cv https://kb-wordpress.gov.cv/kb/portaria-no-73-2005-especificacoes-do-butano/ https://members.wto.org/crnattachments/2025/TBT/CPV/25_03186_00_x.pdf |